Name: Brinton Montgomery

A#: A02472658

Date:

Project 2

Numerical simulation of Compressible Flow

Due 10/31/2025

The purpose of this project is to determine the nature of the shock waves appearing in a numerical simulation of the equations that we are studying in class.

What are the three main equations of compressible flow, from both class and our projects?  Fill in the information below:

Equation 1:

Name: Continuity Equation

Algebraic Form: min=mout→(ρVA)in=(ρVA)out

Differential Form: ∂ρ∂t+∇⋅ρV=0

Equation 2:

Name: Momentum Equation

Algebraic Form:

Differential Form: ∂(ρV)∂t+∇⋅ρVV+∇p=0

Equation 3:

Name: Energy Equation

Algebraic Form:

Differential Form: ∂E∂t+∇⋅[E+pV]=0

The base code for this project is supplied through CANVAS.  Download the CompressibleProject2.mlx and begin to examine the code.  Notice that to simulate our equations on a computer, we need to produce a mesh on which to solve the equations.  Our mesh is an x-y mesh, cartesian coordinate system, that is adjustable in both dimensions.

What are the base number of points in each direction for our simulation?

x-dim number of points __________

y-dim number of points __________

We also must set the actual dimensions of our simulations grid.  To do this we define a variable that tells us the physical size of each grid in our mesh.

What are the x and y physical grid spacings for our grid?

x-grid spacing _____ meters.

y-grid spacing _____ meters.

Total x distance ________ meters.

Total y distance ________ meters.

To solve our equations, we need an equation of state to complete the set of equations.

What is the form of the equation of state for our project?

Equation of state _________

What is the value for the constant in this equation?

Value ____________________

In order to simulate anything of significance with this code, we will have to add an object that we will use in our simulation to determine the impacts from this body on the fluid that we are simulating.

Lines 77 and 78 are where our interaction with this code will start.  The integer variable, item, is used to set the nature (or shape) of our object of interest.  The first item, item = 1, is just a rectangle inserted in the flow moving at some set Mach number.  The Mach number for our simulation is set at line 23 of this code.  The next item available is a wedge at some angle to the horizontal.  The angle is set on line 78.  This angle will be presented early in Chapter 4 of the text.

The way the object is handled in this code, since it is not built on body following coordinates, but instead on a simple cartesian grid, is by producing a mask function.  The mask function in the case of this code, is the full set of grid points, where each value for the mask is either 1 or 0.  The equations are solved if mask(i,j) = 0 and the equations are not solved if mask(i,j) = 1.  This is implemented on line 145 of the code, where we check and only solve the equations where mask(i,j) = 0.

The mask function is set on lines 80-92 for item = 1, and lines 94-113 for item = 2.

NOTE:  The code will only need to be modified at lines 77 and 78 for the majority of this project.

Project Guidelines: Follow these steps.

- Run the code as given.

- Wait for the simulation to end.

- Screen shot the two contourf plots that show the x-direction Mach number, and the pressure in atmospheres.  Toward the bottom of the code, usually.

Place the screen shot in this document:

- Describe in words what is happening in the above screen shot.  Is the shock attached are detached.  What is the Mach number of the flow.  What is the angle of the shock, what would be the θmax for the given Mach number.  What type of reflections are occurring at the top boundary of the simulation?

- Now modify lines 77 to be item = 2.  Rerun the simulation and place the new screen shot in the document below in G.

Place the screen shot in this document:

- Describe in words the difference between screenshots in D and G above.

- Now modify lines 78 to be angle = 10.  Rerun the simulation and place the new screen shot in the document below in G.

Place the screen shot in this document:

- Describe in words the difference between screenshots in D and G above.

One of the main problems with computational solutions is the resolution of the simulations.  Above our simulations are very course due to the limitations of the computers being used.

Leave the code as it was at the end of K above.  Go to line 5 in the code and change the value of factor from 1.0 to 5.0.  This will most likely severely impact the performance of your computer.  Do not worry about this, just find a time when you can leave your computer running for about 12 hours continuously.

This is one means of improving the performance of a simulation when computing power is not a big issue.  When computing power is an issue then we just find a grid spacing that is performable on the computer being used.  In the past computer simulations could take months if not more than a year to complete.

Place the screen shot of the new simulation below:

Describe what the difference in this screen shot is compared to K above, does this make it easier to identify the parts of the solution, specifically the location of the shocks?

For an additional simulation to see the full extent of the usefulness of the code.  Change line 78 to angle = 60.  Now run the code again.  It will take another long period to run, so set aside some time when you will not need your computer for several hours.  This is not part of the project and will not be turned in.  It will show some of the details of what you have learned in class, however.

Specifically compare the last run of the project, the figure just above, to the one produced with this run.  Look closely at the top of the simulation domain, near, x = 6.  What are the differences?  This is what is known as Mach reflection as talked about in class for Chapter 4.